Signal Processing – Home Assignment

### Question A

What is the difference between these graphs?

Answer:
The first graph represents the signal in the time domain, showing how the vibration amplitude changes over time.
The second graph represents the signal in the frequency domain, typically using an FFT, and shows which frequencies are present in the vibration and their relative amplitudes.
While the time-domain graph shows the overall signal behavior, the frequency-domain graph reveals the dominant vibration frequencies and spectral content.

### Question B

What would you do to determine if these are from the same data set?

Answer:
To determine whether both graphs originate from the same data set, I would take the raw time-domain signal used for the first graph and apply the same FFT process to it using identical parameters (sampling rate, windowing, and FFT size).
If the resulting frequency-domain graph matches the second graph, they are from the same data set.
Additionally, I would verify consistency in sampling rate, signal duration, and key characteristics such as dominant frequencies and signal energy.

### Question C

What can you learn about the measured vibration from the second graph that you can't from the first one?

Answer:
From the second graph, it is possible to identify the dominant vibration frequencies, harmonics, and resonance effects that are not easily observable in the time-domain signal.
The frequency-domain graph allows analysis of spectral peaks, noise floor, and frequency distribution, which can provide insights into mechanical faults, imbalance, or periodic behavior that cannot be clearly inferred from the first graph alone.